# LED Nightlight quiz

# 28Jul24 B.R.

## Quiz Instructions

I hope these questions will stimulate further inquiry, understanding, and discussion about the LED nightlight activity. There is no time limit imposed on this quiz. Use whatever resources you can to understand (vs. just answer) the questions and answers.

Top of Form

Question 1

A wall wart measures 12.50VDC open circuit and 12.42VDC across RL = 100.0 ohms, its Rthevenin = (in ohms)

____________

Question 2

In Fig. 3A, R1=1k and the pot R2=10k (more precisely, the left part of it) are in series.

- True

- False

Question 3

A consequences of a resistor's linear I-V curve is that its power is proportional to its current squared.

- True

- False

Question 4

An LED's power is approximately proportional to its current squared.

- True

- False

Question 5

Which of the following are false about R1 in Fig. 3

- R1 protects the LED from burning out

- R1 protects R2 from burning out

- R1 transduces LED current into a proportional voltage

- Reducing R1 to 10 ohms would be fine

Question 6

Again referring to Fig. 3 in the lab, when R2 was set to its middle value (5Kohm), the LED was at its

- minimum current

- a little more than its minimum current

- middle current

- a little less than its maximum current

- maximum current

Question 7

MATLAB calls the operator ./

- point division

- decimal division

- left array division

- stab and slash

- right array division

Question 8

We used another programming language called LaTeX, in this activity, in the command

xlabel('R2 / \Omega');

- True

- False

Question 9

An LED nightlight consists of a 5V (Thevenin voltage) wall wart, Rseries = 1kohm, and the LED has Forward = 2V. If you leave it on constantly, how many kilowatt hours (kWh) will the circuit use in a month?

__________

Question 10

We define efficiency as the ratio of useful power / total power used. LEDs are around 30% efficient in turning electricity into light (i.e., 100 * light power out / electrical power in = .3, which is higher than older lighting technologies). What is the percentage efficiency of the night light circuit in the previous question, i.e., what is 100 * light power output / circuit power input?

_______

Question 11

Our efficiency estimates above could be highly exaggerated because we ignored any internal power consumed by the wall wart itself.

- True

- False

Question 12

Turning the LED off during daytime would have what effect on efficiency

(an ambiguous question. What efficiency? The LED’s efficiency (of turning electricity to light) is what it is … The previous question reminded us much more power is dissipated in the wart, which presumably would remain ON the whole time.

- insignificant

- half it

- double it

- increase it by 4x

Turning the LED off during daytime would have what effect one's electric bill? (Is a less ambiguous question), the answer is A

Bottom of Form

Solutions

- function Rth = rthevenin(Vth, Vl, Rl) … Rth = (Vth - Vl)*Rl ./ Vl;  .644 ohms

- A

- A

- B

- D

- B

- E

- A

- 5V-2V = 3V = VR, I = 3V/1k = 3mA, P = 5V*3mA = 15mW, 15mW*24h/d*30d/m*1W/1e3mW*1kW/1e3W = .0108kWh

- Pwr efficiency = 3mA*2V/(3mA*5V) = 2/5. Times 30% light efficiency = 12%.

- A

- C