# Quiz on Boysen & Kybett Ch. 1 and 2

28Jul24 BR

## Quiz Instructions

6 multiple choice questions based on these 4 drawings. You can use your books and notes, but I hope you don't need them, and this is relatively quick and painless.

Note: this is a timed quiz. You may check the remaining time you have at any point while taking the quiz by pressing the keyboard combination SHIFT, ALT, and T...

Top of Form

Question 1

What does voltmeter V read in Figure 1?

- Vo*R2/(R2+R3)

- Vo*R1(R2+R2)/(R1+R2+R3)

- Vo*R2/(R1+R2+R3)

- Vo*R3/(R1+R2+R3)

- Need a second equation for the second loop

Question 2

Replace R2 in Figure 1 with a capacitor C. Was is V now across the capacitor (in steady-state)?

- 0

- Vo *Zc/(R1+Zc+R3)

- Vo/2

- Vo

- Need another equation for the second loop

Question 3

For the circuit in Fig. 3, what are Vr and Vd, the voltage across the resistor and Si rectifier diode respectively (assuming Vo > Vf=0.7V)?

- Vd = 0V, Vr = Vo

- Vd = 0.7V, Vr = Vo-0.7V

- Vd = Vo-0.7V, Vr = 0.7V

- Vd = Vo, Vr = 0

Question 4

For the circuit in Fig. 4, what are Vr and Vd, the voltage across the resistor and diode respectively (assuming Vo > Vf)?

- Vd = 0V, Vr = Vo

- Vd = 0.7V, Vr = Vo-0.7V

- Vd = Vo-0.7V, Vr = 0.7V

- Vd = Vo, Vr = 0V

Question 5

The sum of the voltages around any circuit (loop) is:

- always zero

- the sum of the active sources in the loop

- ideally zero, but real components may lead to small discrepancies

- depends on what components are in the loop

Question 6

Figure 6 is equivalent to Fig. 2.30 in your book (the light bulb is equivalent to R2). Imagine the light bulb burns out (R2 -> infinity). How does the current through R1 and the Zener diode change?

- IR1 decreases and IZd increases

- IR1 decreases and IZd stays the same

- IR1 decreases and IZd decreases

- IR1 stays the same and IZd increases

- IR1 stays the same and IZd decreases

Bottom of Form

Solutions

- A

- D

- D

- B

- A

- D