# Boysen Ch 3-4 NPN Transistor Quiz

28jul24 BR

## Quiz Instructions

Mini-quiz testing your knowledge of transistors in linear and saturated states. Questions are based on this circuit, hfe = beta = 100. Assume Vbe = .6V when the transistor is on, and the blue LED has Vf=2V.

Note: this is a timed quiz. You may check the remaining time you have at any point while taking the quiz by pressing the keyboard combination SHIFT, ALT, and T...

Question 1

R = 10k, S1 is open, what is VR, the voltage across R, in volts?

Question 2

R = 10k, S1 is now closed, what is VR, the voltage across R, in volts?

Question 3

R = 10k, S1 is open, what is the LED current in mA?

Question 4

R = 10k, S1 is closed, what is the LED current in mA?

Question 5

With S1 closed, as R changes from 1kohm down to 150ohms, the LED will:

- brighten

- dim

- turn off

- stay the same brightness

Question 6

Recall the 4 single transistor circuits Rasnow highlighted. This circuit is a(n):

- amplifier

- current source

- follower

- switch

- Bottom of Form

Solutions

- S1 is open  VR= 0

- VR = 5*10//11 > .6 0.6V

- 0

- I1k= (5-.6)/1k = 4.4.mA, iR = .6V/10k = .06mA, so bIb = 440mA, saturation = (10-2)V/2k =4mA  4mA.

- iR = .6V/150 = 4mA, so bIb = 100*.4 = 40mA, still saturated  no change (D)

- D