# Arduino Quiz

28jul24 BR

## Quiz Instructions

A few questions to test your understanding of the Arduino and our electronics interfaces to it. Unless stated otherwise, assume the Arduino's analog reference pin, Vref = 4.50 VDC.

Question 1

Vref = 4.5V and 3.0VDC is on Arduino pin A0. analogRead(A0) will return what value?

Question 2

The LM317 is adjusted to 10VDC output, and connected directly to Arduino pin A0. analogRead(A0) returns

- 852

- 1023

- 0

- Probably nothing because the Arduino is fried, caput, dead ...

Question 3

The LM317 is adjusted to 8VDC output, and connected to a voltage divider consisting of R1 = 2k and R2 = 1k to ground. What is the voltage across R2 (in volts)?

Question 4

The LM317 is adjusted to 8VDC output, and connected to a voltage divider consisting of R1 = 2k and R2 = 1k to ground. A0 is connected to R2, analogRead(A0) returns what value (in ADU)?

Question 5

The LM317 is adjusted to 8VDC output, and connected to a voltage divider consisting of R1 = 2k and R2 = 1k to ground. A 1kohm load is connected in parallel with the voltage divider and the LM317 (draw a schematic).What is the voltage across R2 (in volts)?

Question 6

The LM317 is adjusted to 8VDC output, and connected to a voltage divider consisting of R1 = 2k and R2 = 1k to ground. A 1kohm load is connected in parallel with R2 of the voltage divider  (draw a schematic). What is the voltage across R2 (in volts)?

Question 7

The LM317 is connected to a voltage divider consisting of R1 = 2k and R2 = 1k to ground, and A0 = the voltage across R2. If analogRead(A0) returns 345, what is the LM317 output voltage (in volts)?

Bottom of Form

Solutions

- 3V * (1023ADU/4.5V) = 682ADU

- D

- V2 = 8V*(1k/1k+2k) = 8/3V = 2.68V

- 2.68V(*1023ADU/4.5V) = 606ADU

- 2.68V

- 8V * .5k/(2k+.5k) = 1.6V

- 345ADU*(4.5V/1023ADU)*(2k+1k)/1k = 4.55V