# Boysen Chapters 5-6 Quiz

28jul24 BR

## Quiz Instructions

Review/study AC circuits as outlined in Ch. 5-6, and Rasnow's class presentations. There are just a few differences, e.g., Rasnow generally doesn't use the term "Reactance". I generalize resistance to impedance, Z = R + jX, where R is the resistance, X is the reactance, and j = sqrt(-1) is the unit complex number. I think of reactance as simply the complex part of the impedance, and I don't give it another name, because I let Matlab do the necessary complex number gymnastics to get to the right answers. The magnitude of an impedance is the magnitude of Z, which in Matlab, is just abs(z).

Also including some questions about Arduino's pulse width modulation (not from Boysen).

For questions asking for numeric answers, just state the number in the requested units, and don't include the unit symbol in your answer. 2 significant figures is enough.

Note: this is a timed quiz. You may check the remaining time you have at any point while taking the quiz by pressing the keyboard combination SHIFT, ALT, and T...

Top of Form

Question 1

A function generator is

- A device for producing transfer functions

- A device for producing AC waveforms

- A device for producing only pure sine waves

- Very expensive

Question 2

A sine wave is measured to have an amplitude of 10Vp-p. Its RMS amplitude would be (in volts)

Question 3

A sine wave is described by V(t) = 12*sin(100*t+30°) where V(t) is in units of volts and t has units of seconds. The RMS voltage (in volts) would be:

Question 4

The same voltage, V(t) = 12*sin(100*t+30°), would have a measured frequency (in Hertz) of

Question 5

The same voltage, V(t) = 12*sin(100*t+30°), would have a period (in msec):

Question 6

The following circuit relates to this and the following 2 questions.

The 10kohm resistor at 1kHz has impedance magnitude (in kohms) of

Question 7

The 10nf capacitor has impedance magnitude (in kohms) of

Question 8

What frequency (in kHz) are the impedance magnitudes of R = 10kohm and C = 10nf equal?

Question 9

An R-C filter consists of 2kohms in series with 4.7uf. What is its corner frequency (in Hz)?

Question 10

Assume your Arduino has Vref = 4.5V. analogWrite(5, 52) will produce what DC voltage (answer in millivolts) on pin D5?

Question 11

Assume your Arduino has Vref = 4.5V. You wish to produce a DC voltage of 2.0V using analogWrite. What PWM value would you use?

Bottom of Form

Solutions

- B

- 5/sqrt(2) = 3.5V

- 12/sqrt(2) = 8.5

- 2*pi*f = 100Hz  f = 16

- 1/16*1000msec/sec = 63

- 10

- zc(10e-9,1e3) = 16kohms = 16

- f = 1/(2*pi*zc*c) = 1/(2*pi*10e3*10e-9) = 1.59kHz

- fc = 1/(2*pi*R*C) = 17Hz

- 4.5V * 52/255 = .917V = 917mV

- 2.0V * 255PWMU/4.5V = 113 PWMU