# LM317 quiz

## Instructions

Answer the following questions based on the LM317 circuit below, the LM317 data sheet, and the following assumptions:

a) For the NPN transistors, assume Vbe saturation = 0.6V, Vce saturation = 0.1V, and b = 200

b) AREF = 4.6V

c) Resistors may have different values than in your physical circuit. Use the values in the enclosed diagram.

I strongly encourage you NOT to guess ... intuition is a terrible guide in this jungle, and you'll have much better success following the paths of Masters Ohm, Kirchhoff, Voltage Divider Equation, characteristics of the devices, etc.

## Unit conversion

What value will analogRead(A0) return when DC Vout = 9V?

142

667

840

1000

1840

What will be the voltages on pin D5 following analogWrite(5, 180);?

- 0.81VDC and 0VAC

0.81VDC and 2.3VAC

0.81VDC and 2.5VAC

3.25VDC and 0VAC

3.25VDC and 2.3VAC

## LM317

The LM317 datasheet says the voltage between the OUT and ADJ pin is 1.25V. How much current flows through R1?

- 100uA

470uA

1mA

2.7mA

4.7mA

When the LM317 is outputting its maximum voltage, R2 is at its minimum value.

- True

False

How much current flows through R2 when it's in the middle of its range (5K)?

- 100uA

470uA

1mA

2.7mA

4.7mA

What's the voltage across R2 when it is in the middle of its range, assuming Q3 is off (Ic~10uA leakage)?

- 4.4V

6.6V

8.8V

11V

Assuming Q3 is off (Ic~10uA leakage), what is DC Vout when R2 is in the middle of its range?

- 4V

6V

8V

10V

## PWM filter

The corner frequency of the first stage of the PWM filter is

- 7Hz

80Hz

980Hz

10kHz

The corner frequency of the 2nd stage of the PWM filter is

- 7Hz

80Hz

980Hz

10kHz

If analogWrite set the voltage on pin D5 to 3.0VDC, then the steady-state voltage on the base of Q1 will be

- 0.14VDC and 0VAC

.14VDC and 2.5VAC

2.4VDC and 0VAC

3.0VDC and 0VAC

3VDC and 2.5VAC

## Emitter follower

If the base of Q1 is 3.0VDC, then the emitter of Q1 will be at

- 0VDC

0.6VDC

2.4VDC

3.0VDC

3.6VDC

## Putting it all together

Extra credit: If R2 = 5kohm and Q3's collector current is 0.95mA, what is DC Vout?

- 4V

6V

8V

10V

Extra credit: What do you predict DC Vout would be for PWM = 192?

- 4

6

8

1